Product Specification Document

Issue: 1

Date 02/20				Product Code:	 INXJ

Product Name & Description:	 LDPE BLUE Sacks			Page: 1 of 2

| Description | Unit | Data | Tolerance |
| --- | --- | --- | --- |
| Closed Width | mm | 457 | +/- 2.5% |
| Open Width | mm | 737 | +/- 2.5% |
| Length | mm | 965 | +/- 2.5% |
| CHSA Min Nett Weight | kg | 4.73 |  |
| Film Colour | na | BLUE |  |
| Opacity | % | 80 |  |
| Blend Requirement | na | 100% Recycled LDPE |  |
| Print Requirement |  | NA | No Ink Lift |
| Handle Requirement | Yes/No Style | No |  |
| Carton Qty |  | 200 |  |

Film Performance:

| Description | Unit | Data | Tolerance |
| --- | --- | --- | --- |
| CHSA Ultimate Lift | Kg | 15Kg (Medium Duty ) | Manual Test |
| Seals |  | No Burst under air pressure | Manual Test |
| Visual inspection |  | Free from visual defects, and of consistent colour and finish. | Manual Test |
| Other |  | Sacks to be free from strong odour and demonstrate no blocking. | Manual Test |

Product Name & Description:		LDPE Black Sacks			Page: 2 of 2

Carton Details:

| Description | Unit | Data | Tolerance |
| --- | --- | --- | --- |
| Width | mm | 265 external | max |
| length | mm | 470 external | max |
| Height | mm | 100 external | max |
| Card type | Flue | Brown Outer Card, 150 kraft test, B flute, 3 ply | min |
| Label / Print |  | Printed 1 colour carton |  |
| Pallet Quantity | Count | 88 cartons (11 layers x 8) |  |
| Carton Strength |  | Carton must not crush under min of 12x weight. Bulging Cartons not acceptable. |  |

CARTON IMAGE:

| Approved By | Date | Signature |
| --- | --- | --- |
| Angela Kuzmic |  |  |